Firewall setting for Videoconferencing

Introduction

The function of a “Firewall” is to protect a private network from unauthorized traffic entering from the Internet.  A “Firewall” is software that resides on a server.  The firewall is electronically between the “Edge Router” and the rest of the private network.  The firewall allows access to the local network by opening “Ports”.

Port Assignments

While communicating over an Internet Protocol (IP) network using either Transport Control Protocol (TCP) or User Datagram Protocol (UDP), the originating device assigns a Port Number to be used for the communication session.

The available ports numbers are 0 to 65535.  They are divided into three categories.  Port numbers 0 through 1023 are known as “Well Known Port Numbers”.  They have been assigned by the Internet Corporation for Assigned Names and Numbers (ICANN) to be used for well know applications such as File Transfer Protocol (FTP).

Numbers from 1024 to 49151 can be registered by organizations for specific applications.  If an entity registers a port number or numbers for an application, the organization will use that number and advise end users to open the port or ports on their firewall to allow the use of this application.  Companies can allow their products to dynamically select available ports from a range of numbers in this category.  The entity must then instruct end users to allow communication on these ports to traverse their firewall.

Numbers from 49152 through 65535 can be dynamically assigned by end points, but can not be reserved for any specific application.  Once again, an organization that uses ports from this group must advise the end user of the range of ports the firewall should hold open.

TCP vs. UDP

TCP is a transport layer (Layer 4) protocol that incorporates the acknowledgement of received packets and the retransmission of lost packets.  This adds latency to the end-to-end communication, but insures the integrity of the transaction.  UDP does not provide for either packet acknowledgement or retransmission within the transaction layer.  The application can make provision for verification and retransmission.  UDP, while less reliable, is faster.

The ports discussed above can be used for either TCP or UDP packet transmissions.

Well Known Port Numbers Used in Videoconferencing

| Port | Type | Protocol | Application | Manufacturer |
| --- | --- | --- | --- | --- |
| 21 | Static | TCP | File Transfer Protocol for endpoint software upgrades (must be bi-directional) | Polycom and Tandberg |
| 23 | Static | TCP & UDP | Telnet (must be bi-directional) | Polycom, Sony, Tandberg |
| 80 | Static | TCP | Hypertext Transfer Protocol (HTTP) - web browser interface for codec control and menus | Polycom, Sony, Tandberg |
| 161 | Static | UDP | Simple Network Management Protocol (SNMP) Queries | Tandberg |
| 389 | Static | TCP | Lightweight Directory Access Protocol (LDAP) – ILS registration | Polycom |
| 962 | Static | UDP | Simple Network Management Protocol (SNMP) Traps | Tandberg |
| 963 | Static | TCP | This port is not assigned, but Tandberg uses it for Netlog | Tandberg |
| 964 | Static | TCP | This port is not assigned, but Tandberg uses it for FTP/data | Tandberg |
| 965 | Static | TCP | This port is not assigned, but Tandberg uses it for VNC | Tandberg |
| 970 | Static | UDP | This port is not assigned, but Tandberg uses it for Real-time Transport Protocol (RTP) for streaming video | Tandberg |
| 971 | Static | UDP | This port is not assigned, but Tandberg uses it for Real-time Transport Control Protocol (RTCP) for streaming video | Tandberg |
| 972 | Static | UDP | This port is not assigned, but Tandberg uses it for Real-time Transport Protocol (RTP) for streaming audio | Tandberg |
| 973 | Static | UDP | This port is not assigned, but Tandberg uses it for Real-time Transport Control Protocol (RTCP) for streaming audio | Tandberg |
| 974 | Static | UDP | This port is not assigned, but Tandberg uses it for SAP | Tandberg |
| 1002 | Static | UDP | This port is not assigned, but Vcon uses it for Lightweight Directory Access Protocol (LDAP) – ILS registration | Vcon |

Registered Port Numbers Used in Videoconferencing

| Range | Type | Protocol | Application | Manufacturer |
| --- | --- | --- | --- | --- |
| 1300 | Static | TCP & UDP | This port is registered to Intel and is used to secure a H.323 host call – h 323hostcsllsc (must be bi-directional) | Polycom |
| 1503 | Static | TCP | This port is registered to Databeam and is used for T.120 file sharing | Polycom, Sony, Tandberg and Vcon |
| 1718 | Static | TCP & UDP | This port is registered to Intel and is used to secure a H.323 host call – h 323gatekeepeer discovery (must be bi-directional) | Polycom, Sony, and Vcon |
| 1719 | Static | TCP & UDP | This port is registered to Intel and is used for gatekeeper RAS – h 323gatestat (must be bi-directional) | Polycom, Sony, Tandberg and Vcon |
| 1720 | Static | TCP & UDP | This port is registered to Intel and is used to establish a H.323 host call using Q.931 call setup – h 323hostcall (must be bi-directional) | Polycom, Sony, Tandberg and Vcon |
| 1731 | Static | TCP & UDP | Audio call control –msiccp – for VoIP | Polycom |
| 1024 - 65535 |  |  |  | Vcon |
| 2253 - 2255 | Dynamic |  | Sony uses an available port in this range for the exchange of H.245 call parameters. (Also known as RTCP) | Sony |
| 2326 - 2373 | Dynamic | UDP | Tandberg uses an available port in this range for video data streams | Tandberg |
| 2326 - 2373 | Dynamic | UDP | Tandberg uses an available port in this range for audio data streams | Tandberg |
| 2326 - 2373 | Dynamic | UDP | Tandberg uses an available port in this range for data transfers and Far End Camera Control - FECC | Tandberg |
| 2979 | Static | TCP & UDP | This port is registered to ACM for H.263 Video Streaming | Polycom |
| 3230 - 3247 | Dynamic | UDP | Polycom uses an available ports in this range for audio and video | Polycom |
| 3230 - 3235 | Dynamic | UDP | Polycom uses an available port in this range for the exchange of H.245 call parameters. (Also known as RTCP) | Polycom |
| 5004 - 6004 | Dynamic | TCP | There is no registered port for this application, Vcon uses an available port for H.245 (Call Parameters) | Vcon |
| 5004 - 6004 | Dynamic | UDP | There is no registered port for this application, Vcon uses an available port for Real-time Transport Protocol (RTP) for streaming video. | Vcon |
| 5004 - 6004 | Dynamic | UDP | There is no registered port for this application, Vcon uses an available port for Real-time Transport Protocol (RTP) for streaming audio. | Vcon |
| 5004 - 6004 | Dynamic | UDP | There is no registered port for this application, Vcon uses an available port for Real-time Transport Control Protocol (RTCP) for streaming video and audio. | Vcon |
| 5555-5556 | Dynamic | TCP | Q.931 Call setup | Tandberg |
| 11720 | Static | TCP & UDP | This port is registered to Cisco and is used as an alternative for call set-up – h 323hostcallsigalt (must be bi-directional) | Polycom |
| 22136 | Static | TCP | There is no registered port for this application, Vcon uses an available port for remote Vcon endpoint administration | Vcon |
| 26505 | Static | TCP | There is no registered port for this application, Vcon uses an available port for Remote Console | Vcon |

Other Port Numbers Used in Videoconferencing

| Range | Type | Protocol | Application | Man. |
| --- | --- | --- | --- | --- |
| 49152 - 49159 | Dynamic | UDP | Sony uses this range of ports for audio and video data streams | Sony |
| 49152 - 49239 | Dynamic | UDP | Sony uses this range of ports for multipoint | Sony |
| 49195 | Static | UDP | SIP TLS Transfer | Tandberg |

Polycom

| Port | Type | Protocol | Description |
| --- | --- | --- | --- |
| 224.0.1.41:1718 | Static | TCP & UDP | h323gatekeeper discovery (must be bi-directional) |
| 1719 | Static | TCP & UDP | h323 gatekeeper RAS (must be bi-directional) |
| 1720 | Static | TCP & UDP | h323hostcall Q.931 (Call Setup) (must be bi-directional) |
| 1731 | Static | TCP & UDP | msiccp Audio Call Control (VoIP) |
| 3230 - 3247 | Dynamic | UDP | Audio and Video (must be bidirectional) |
| 3230 - 3235 | Dynamic | TCP | H.245 call control: aka RTCP (must be bidirectional) |
| Other: |  |  |  |
| Port | Type | Protocol | Description |
| 21 | Static | TCP | FTP allows upgrade of endpoint software (must be bidirectional) |
| 23 | Static | TCP | Telnet (must be bidirectional) |
| 80 | Static | TCP | Web browser interface to codec controls and menus |
| 389 | Static | TCP | ILS Registration (LDAP) |
| 1300 | Static | TCP & UDP | h323hostcsllsc H323 Host Call Secure |
| 1503 | Static | TCP & UDP | T.120 (Data Channel in a multipoint) |
| 2979 | Static | TCP & UDP | H.263 Video Streaming |
| 11720 | Static | TCP & UDP | h323callsigalt H.323 Call Signal Alternate |

Sony PCS – X

| Port | Type | Protocol | Description |
| --- | --- | --- | --- |
| 1718 | Static | TCP | h323gatekeeper discovery (must be bi-directional) |
| 1719 | Static | TCP | h323gatestat |
| 1720 | Static | TCP | H323hostcall |
| 2253 - 2255 | Dynamic | TCP | H.245(Call Parameters) |
| 49152- 49159 | Dynamic | UDP (RTP/RTCP) | Audio & Video Data Streams |
| 49152 - 49239 | Dynamic | UDP | Multipoint |

Tandberg

| Port | Type | Protocol | Description |
| --- | --- | --- | --- |
| 1719 | Static | UDP | Gatekeeper RAS |
| 1720 | Static | TCP | Q.931 (Call Setup) |
| 5555 - 5556 | Dynamic | TCP | H.245(Call Parameters) |
| 2326- 2373 | Dynamic | UDP | Video Data Streams |
| 2326- 2373 | Dynamic | UDP | Audio Data Streams |
| 2326- 2373 | Dynamic | UDP | Data/FECC |
| 21 | Static | TCP | FTP |
| 80 | Static | TCP | HTTP |
| 123 | Static | UDP | NTP |
| 161 | Static | UDP | SNMP (Queries) |
| 962 | Static | UDP | SNMP (Traps) |
| 963 | Static | TCP | Netlog |
| 964 | Static | TCP | FTP/data |
| 965 | Static | TCP | VNC |
| 970 | Static | UDP | Streaming/RTP Video |
| 971 | Static | UDP | Streaming/RTCP Video |
| 972 | Static | UDP | Streaming/RTP Audio |
| 973 | Static | UDP | Streaming/RTCP Audio |
| 974 | Static | UDP | SAP (Stream is directed to 224.2.127.254:9875) |
| 5060 | Static | TCP | Movi Call Setup |
| 21000-50000 | Dynamic | UDP | Movi Communication ports (4 ports) |
| 5082 | Static | UDP | ConferenceMe |
| 4444 | Static | TCP | Movi Multiway Call Setup |

Vcon

| Port | Type | Protocol | Description |
| --- | --- | --- | --- |
| 1718 | Static | UDP | h323gatekeeper discovery (must be bi-directional) |
| 1719 | Static | UDP | gatekeeper RAS |
| 1720 | Static | TCP | Q.931 (Call Setup) |
| 5004 - 6004 | Dynamic | TCP | H.245(Call Parameters) |
| 5004 - 6004 | Dynamic | UDP (RTP) | Video Data Streams |
| 5004 - 6004 | Dynamic | UDP (RTP) | Audio Data Streams |
| 5004 - 6004 | Dynamic | UDP (RTCP) | Control Information |
| Optional: |  |  |  |
| Port | Type | Protocol | Description |
| 389 | Static | TCP | ILS Registration (LDAP) |
| 1002 | Static | TCP | Site Server Registration (Windows 2000 Built-in LDAP) |
| 1503 | Static | TCP | T.120 (Data Channel) |
| 22136 | Static | TCP | VCON MXM - Remote VCON Endpoint Admin |
| 26505 | Static | TCP | VCON MXM - Remote Console |

The preferred method of dealing with firewalls is to bypass them when possible or appropriate. UEN has some tried and proven methods for bypass. The Network Engineering staff should be consulted to see what method should be tried.

It may be necessary to provide trusted network information to facilitate getting endpoints to communicate with TMS, gatekeeper and various UEN testing systems. Here is the information that can be provided to firewall and network admins. This information provides access for TMS, our local codecs and workstations.

205.127.232.0 255.255.255.0

205.127.233.0 255.255.255.0

205.127.238.0 255.255.255.0

205.127.225.40 255.255.255.0

205.127.227.0 255.255.255.0

140.197.0.0 255.255.255.0

140.197.5.0 255.255.255.0

140.197.225.162 255.255.255.0